Intermuscular coherence during robot-aided isometric wrist tasks: The role of wrist posture and interaction force

Thesis Student: Pranav Balasundaram

Program: MS in Electrical Engineering

Thesis Director and Chair:

Andrew Erwin, Mechanical Engineering

Secondary appointment in ECE

Thesis Work

This thesis presents an experimental protocol that facilitates computing intermuscular coherence (IMC) in the human wrist during an isometric force task. The task uses a benchtop two degrees-of-freedom wrist exoskeleton to standardize and measure the application of isometric force. By doing so, the experimental protocol can evaluate whether strength and frequency of IMC changes with wrist posture, muscle force, and force application. The experimental implementation includes an interactive ‘game’ in which an individual applies isometric forces at specific `targets' represented along the wrist flexion/extension and radial/ulnar deviation axes. Participants interact with the robotic device by gripping a handle attached to the robot end effector while their forearm is secured with fabric straps. The participant is equipped with electromyography sensors on the arm prior to completing the task. Ultimately, the experimental protocol provides an important first step in developing a biomarker that can quantify changes in motor control after neurological injuries, such as stroke.

Thesis Background

The wrist plays an essential role in using the hand during physical interactions with the environment. From body weight movements to everyday physiological forces/loads, the forces experienced by the arm/hand during these interactions are essentially borne by the wrist. Consequently, the wrist also plays an important role in the stabilization/load transfer of the hand. Importantly, generating torque at the wrist requires coordination of muscles in the forearm and upper arm. The purpose of this thesis is to develop an experimental protocol that enables quantification of the extent to which arm muscles are synchronized when using the wrist. The humans subject experimental protocol relies on a robotic assessment device, electromyography sensors, and a GUI.

Technical Background (Intermuscular Coherence):

Shared neural drive to groups of muscles contains a distinct frequency component from oscillatory neural circuits in the brain that generate the neural inputs for voluntary motor control. The strength of shared neural drive between two muscles can be quantified across the frequency spectrum through intermuscular coherence (IMC) analysis between two processed electromyography (EMG) signals. IMC can quantify the strength of shared neural drive to muscles both in healthy function and dysfunction. As a result, IMC is a promising biomarker of motor impairment after neurological injuries, such as stroke. Specifically, IMC quantifies the degree of synchrony between two signals using magnitude squared coherence analysis (referred to simply as coherence). Coherence analysis can be used to estimate the amount of correlation between two signals as a function of their frequencies. In other words, coherence in the frequency domain is what correlation is to the time domain. It computes values ranging from 0 to 1, with higher values corresponding to a higher degree of synchronization. In summary, IMC analysis is a promising (albeit indirect) means to estimate shared neural drive and as such is a promising biomarker of upper extremity impairments.